📘 Lab Report

Configure IPv6 Addresses on Network Devices

Lab 12.9.2 - Cisco CCNA

🔧 Cisco IOS     🌐 IPv6     📶 Networking     ✅ Completed

## 📋 Table of Contents

1. Executive Summary

2. Network Topology & Addressing Plan

3. Configuration Steps

4. Verification & Testing

5. Key Concepts & Justifications

6. Troubleshooting Guide

7. Conclusion

# 1️⃣ Executive Summary

Lab Objective:

Configure a dual-LAN IPv6 network with manual addressing, enable SLAAC (Stateless Address Autoconfiguration), and verify end-to-end connectivity between hosts on different subnets through a Cisco router.

Key Technologies:

- IPv6 Global Unicast Addressing (2001:db8::/32 documentation prefix)

- IPv6 Link-Local Addressing (fe80::/10)

- SLAAC (Stateless Address Autoconfiguration via Router Advertisements)

- Multicast Groups (FF02::1, FF02::2, Solicited-Node)

- Switch SDM Template (dual-ipv4-and-ipv6)

Equipment Used:

- 1x Cisco Router 4221 (or equivalent) - R1

- 1x Cisco Catalyst 2960 Switch - S1

- 2x PCs (PC-A, PC-B)

⏱️ Completion Time: ~90 minutes

# 2️⃣ Network Topology & Addressing Plan

## 2.1 Network Diagram

PC-B (.a::3)  ←→  [G0/0/0] R1 [G0/0/1]  ←→  S1  ←→  PC-A (.1::3)
         ↓                              ↓              ↓
    Network A                      Network 1      VLAN 1 (.1::b)
  (2001:db8:acad:a::/64)      (2001:db8:acad:1::/64)

## 2.2 Complete Addressing Table

| Device | Interface | IPv6 Address | Prefix | Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 2001:db8:acad:a::1 | /64 | N/A |
| R1 | G0/0/1 | 2001:db8:acad:1::1 | /64 | N/A |
| R1 | Both | fe80::1 (link-local) | /10 | N/A |
| S1 | VLAN 1 | 2001:db8:acad:1::b | /64 | fe80::1 |
| PC-A | NIC | 2001:db8:acad:1::3 | /64 | fe80::1 |
| PC-B | NIC | 2001:db8:acad:a::3 | /64 | fe80::1 |

📝 Note: The 2001:db8::/32 prefix is reserved for documentation purposes per RFC 3849.

# 3️⃣ Configuration Steps

## 3.1 Part 1: Basic Device Configuration

### Step 1: Router R1 Initial Setup

! Enter privileged EXEC mode
Router> enable
Router# configure terminal
! Set hostname
Router(config)# hostname R1
! Secure privileged EXEC mode
R1(config)# enable secret class
! Configure MOTD banner
R1(config)# banner motd $Unauthorized access is prohibited!$
! Secure console line
R1(config)# line console 0
R1(config-line)# password ccna
R1(config-line)# login
R1(config-line)# exit
! Secure VTY lines (Telnet/SSH)
R1(config)# line vty 0 4
R1(config-line)# password ccna
R1(config-line)# login
R1(config-line)# exit
! Encrypt all plaintext passwords
R1(config)# service password-encryption
! Disable DNS lookups
R1(config)# no ip domain-lookup
R1(config)# end

💡 Justification:

- enable secret uses MD5 hashing (type 5), more secure than enable password

- service password-encryption applies type 7 encryption to plaintext passwords

- no ip domain-lookup prevents delays from DNS resolution on typos

- banner motd provides legal warning before login (compliance requirement)

### Step 2: Switch S1 Initial Setup

! Same basic configuration as R1
Switch> enable
Switch# configure terminal
Switch(config)# hostname S1
S1(config)# enable secret class
S1(config)# banner motd $Unauthorized access is prohibited!$
S1(config)# line console 0
S1(config-line)# password ccna
S1(config-line)# login
S1(config-line)# exit
S1(config)# line vty 0 4
S1(config-line)# password ccna
S1(config-line)# login
S1(config-line)# exit
S1(config)# service password-encryption
S1(config)# no ip domain-lookup
S1(config)# end

### ⚠️ CRITICAL Step 3: Enable IPv6 Support on Switch

! Check current SDM template
S1# show sdm prefer
! Configure dual IPv4/IPv6 template
S1# configure terminal
S1(config)# sdm prefer dual-ipv4-and-ipv6 default
S1(config)# end
! Reload required for template change
S1# reload
Proceed with reload? [confirm] <Enter>
! Wait 1-2 minutes for switch to reboot

🔴 CRITICAL: The default Catalyst 2960 SDM template does NOT support IPv6. Without changing to dual-ipv4-and-ipv6 and reloading, you CANNOT assign IPv6 addresses to VLAN 1 SVI. This step is mandatory!

## 3.2 Part 2: IPv6 Address Configuration

### Step 4: Configure IPv6 on R1 Interfaces

Sub-step A: Assign Global Unicast Addresses

R1# configure terminal
! Configure G0/0/0 (Network A)
R1(config)# interface gigabitEthernet 0/0/0
R1(config-if)# ipv6 address 2001:db8:acad:a::1/64
R1(config-if)# no shutdown
! Configure G0/0/1 (Network 1)
R1(config-if)# interface gigabitEthernet 0/0/1
R1(config-if)# ipv6 address 2001:db8:acad:1::1/64
R1(config-if)# no shutdown
R1(config-if)# end

Sub-step B: Verify Addresses

R1# show ipv6 interface brief
GigabitEthernet0/0/0   [up/up]
    FE80::D68C:B5FF:FECE:A0C0     <- Auto-generated link-local
    2001:DB8:ACAD:A::1             <- Our global unicast
GigabitEthernet0/0/1   [up/up]
    FE80::D68C:B5FF:FECE:A0C1
    2001:DB8:ACAD:1::1

💡 Observation: Link-local addresses (fe80::) are auto-generated using EUI-64 (based on interface MAC address). Each interface gets a unique link-local by default.

Sub-step C: Configure Manual Link-Local Addresses

R1# configure terminal
R1(config)# interface g0/0/0
R1(config-if)# ipv6 address fe80::1 link-local
R1(config-if)# interface g0/0/1
R1(config-if)# ipv6 address fe80::1 link-local
R1(config-if)# end

💡 Why Same Link-Local on Both Interfaces? Link-local packets NEVER leave the local segment (non-routable). Since each interface belongs to a different network segment, there's no conflict. This simplifies configuration - both use fe80::1 as default gateway.

Sub-step D: Verify Link-Local Change

R1# show ipv6 interface g0/0/0
GigabitEthernet0/0/0 is up, line protocol is up
  IPv6 is enabled, link-local address is FE80::1  <- Changed!
  Global unicast address(es):
    2001:DB8:ACAD:A::1, subnet is 2001:DB8:ACAD:A::/64
  Joined group address(es):
    FF02::1          <- All-nodes multicast
    FF02::1:FF00:1   <- Solicited-node multicast

📋 Lab Question:

Q: Which two multicast groups have been assigned to interface G0/0/0?

A: FF02::1 (all-nodes) and FF02::1:FF00:1 (solicited-node for address ::1)

### Step 5: Enable IPv6 Routing & SLAAC

Sub-step A: Check PC-B Before Routing Enabled

On PC-B command prompt:

C:\> ipconfig
! Result: NO IPv6 global unicast address yet

Sub-step B: Enable IPv6 Unicast Routing

R1# configure terminal
R1(config)# ipv6 unicast-routing  <- This is THE key command!
R1(config)# exit

🔥 CRITICAL: This single command transforms R1 into an IPv6 router. It:
   1) Enables IPv6 routing between interfaces
   2) Joins the FF02::2 (all-routers) multicast group
   3) Starts sending Router Advertisements (RA) with network prefix
   4) Enables hosts to use SLAAC for auto-configuration

Sub-step C: Verify FF02::2 Group Membership

R1# show ipv6 interface g0/0/0
Joined group address(es):
    FF02::1      <- All-nodes (was already there)
    FF02::2      <- All-routers (NEW! Router role activated)
    FF02::1:FF00:1

Sub-step D: Verify SLAAC on PC-B

On PC-B, run ipconfig again:

C:\> ipconfig
IPv6 Address: 2001:db8:acad:a:xxxx:xxxx:xxxx:xxxx  <- SLAAC address!
Default Gateway: fe80::1                            <- From RA

📋 Lab Question:

Q: Why did PC-B receive the Global Routing Prefix and Subnet ID from R1?

A: R1 now sends Router Advertisements (RA) to FF02::1 (all nodes) containing:
   • Network prefix: 2001:db8:acad:a::/64
   • Default gateway: fe80::1 (R1's link-local)
   • SLAAC flag enabled
PC-B receives the RA, extracts the /64 prefix, generates its own Interface ID (64 bits using privacy extensions or EUI-64), and combines them to create a full global unicast address. This is SLAAC (Stateless Address Autoconfiguration) - no DHCP server needed!

### Step 6: Configure IPv6 on Switch S1

S1# configure terminal
! Configure management interface (SVI)
S1(config)# interface vlan 1
S1(config-if)# ipv6 address 2001:db8:acad:1::b/64
S1(config-if)# ipv6 address fe80::b link-local
S1(config-if)# no shutdown
S1(config-if)# end

Verification:

S1# show ipv6 interface vlan 1
Vlan1 is up, line protocol is up
  IPv6 is enabled, link-local address is FE80::B
  Global unicast address(es):
    2001:DB8:ACAD:1::B, subnet is 2001:DB8:ACAD:1::/64
  Joined group address(es):
    FF02::1
    FF02::1:FF00:B

💡 Note: VLAN 1 is the default management interface on switches. We configure IPv6 here to manage the switch remotely via SSH or to ping it from other devices.

### Step 7: Configure Static IPv6 on PCs

On PC-A:

Desktop → IP Configuration → IPv6 Configuration:

- IPv6 Address: 2001:db8:acad:1::3

- Prefix Length: 64

- Default Gateway: fe80::1

On PC-B:

- IPv6 Address: 2001:db8:acad:a::3

- Prefix Length: 64

- Default Gateway: fe80::1

💡 Important: Each PC will now have TWO global IPv6 addresses:
   1) Static address (manually configured above)
   2) SLAAC address (auto-generated from R1's RA messages)
This is normal and expected in IPv6 - multiple addresses per interface is a feature, not a bug!

# 4️⃣ Verification & Testing

## 4.1 Connectivity Tests from PC-A

### Test 1: Ping R1 Link-Local

C:\> ping fe80::1
Pinging fe80::1 with 32 bytes of data:
Reply from fe80::1: time<1ms
Reply from fe80::1: time<1ms
✅ SUCCESS

Validates: PC-A ↔ R1 local connectivity on Network 1

### Test 2: Ping S1 Management Interface

C:\> ping 2001:db8:acad:1::b
Pinging 2001:db8:acad:1::b with 32 bytes of data:
Reply from 2001:db8:acad:1::b: time<1ms
✅ SUCCESS

Validates: PC-A can reach S1's IPv6 management interface

### Test 3: Traceroute to PC-B

C:\> tracert 2001:db8:acad:a::3
Tracing route to 2001:db8:acad:a::3 over a maximum of 30 hops:
  1   <1 ms   <1 ms   <1 ms   2001:db8:acad:1::1   [R1-G0/0/1]
  2   <1 ms   <1 ms   <1 ms   2001:db8:acad:a::3   [PC-B]
Trace complete.
✅ SUCCESS - End-to-end connectivity confirmed!

Validates: IPv6 routing is working between Network 1 and Network A through R1

## 4.2 Connectivity Tests from PC-B

### Test 4: Ping PC-A

C:\> ping 2001:db8:acad:1::3
✅ SUCCESS

### Test 5: Ping R1 Link-Local (G0/0/0 side)

C:\> ping fe80::1
✅ SUCCESS

🎯 All Tests Passed! Full IPv6 connectivity established across both networks.

# 5️⃣ Key Concepts & Justifications

## 5.1 Link-Local Address Reuse

📋 Reflection Question 1:

Q: Why can the same link-local address (fe80::1) be assigned to both Ethernet interfaces on R1?

A: Link-local addresses (fe80::/10) are designed for communication ONLY within a single network segment. Packets with link-local source or destination addresses are NEVER routed beyond the local link. Since G0/0/0 and G0/0/1 belong to completely separate network segments (Network A and Network 1), there is no possibility of address conflict. This is a best practice in IPv6 - using fe80::1 as the default gateway on all router interfaces simplifies configuration and is easy for users to remember.

## 5.2 IPv6 Address Structure

Example: 2001:db8:acad:1::3/64

| 2001:db8:acad | Global Routing Prefix (48 bits - /48) |
| --- | --- |
| 1 | Subnet ID (16 bits - 4th hextet) |
| ::3 | Interface ID (64 bits - last 64 bits) |
| /64 | Prefix Length (64 bits for network portion) |

📋 Reflection Question 2:

Q: What is the Subnet ID of the IPv6 address 2001:db8:acad::aaaa:1234/64?

A: 0 (or 0000). 
Explanation: In an IPv6 address with /64 prefix, the Subnet ID is the 4th hextet. In 2001:db8:acad::aaaa:1234, the double colon (::) represents zero compression. Expanded, this address is:
2001:db8:acad:0000:0000:0000:aaaa:1234
The 4th hextet (Subnet ID) = 0000, so the network prefix is 2001:db8:acad:0::/64 (or in compressed form: 2001:db8:acad::/64)

## 5.3 SLAAC (Stateless Address Autoconfiguration)

How SLAAC Works:

- Host generates link-local address (fe80:: + interface ID)

- Host listens for Router Advertisements (sent by router to FF02::1)

- Host extracts /64 prefix from RA (e.g., 2001:db8:acad:a::/64)

- Host generates Interface ID (64 bits) using:
   • EUI-64 (based on MAC address), OR
   • Privacy extensions (random)

- Host combines: Prefix (64 bits) + Interface ID (64 bits) = Complete global unicast address

- Host uses router's link-local as default gateway

🎯 Result: No DHCP server needed for basic IPv6 addressing!

## 5.4 IPv6 Multicast Groups

| Group Address | Purpose |
| --- | --- |
| FF02::1 | All IPv6 nodes on the local link |
| FF02::2 | All IPv6 routers (enables RA transmission) |
| FF02::1:FFxx:xxxx | Solicited-node (used for Neighbor Discovery - replaces ARP) |

# 6️⃣ Troubleshooting Guide

## Common Issues & Solutions

| ❌ | Problem | Solution |
| --- | --- | --- |
| 1 | Cannot assign IPv6 to S1 VLAN 1 | sdm prefer dual-ipv4-and-ipv6 default + reload |
| 2 | PCs not getting SLAAC address | Enable ipv6 unicast-routing on R1 |
| 3 | Interface shows [admin down] | Use no shutdown on the interface |
| 4 | No inter-VLAN routing | Verify R1 has correct /64 on both interfaces |
| 5 | PC can't ping default gateway | Check PC's gateway is fe80::1, verify link-local on R1 |

Verification Commands:

! On Router
show ipv6 interface brief
show ipv6 interface g0/0/0
show ipv6 route
! On Switch
show sdm prefer
show ipv6 interface vlan 1
! On PC
ipconfig
ping <address>
tracert <address>

# 7️⃣ Conclusion

Lab Achievements:

- Successfully configured IPv6 global unicast addresses on router interfaces

- Implemented manual link-local addressing for simplified gateway configuration

- Enabled IPv6 routing with ipv6 unicast-routing

- Configured SLAAC functionality via Router Advertisements

- Properly configured Switch SDM template for IPv6 support

- Assigned static IPv6 addresses to end hosts

- Verified full end-to-end IPv6 connectivity with ping and traceroute

Key Takeaways:

- IPv6 /64 prefix split: First 64 bits = network, last 64 bits = host

- Link-local reusability: Same fe80:: address can exist on multiple interfaces (non-routable)

- SLAAC eliminates DHCP: Hosts auto-configure using Router Advertisements

- Switch SDM template critical: Must enable dual-stack support before IPv6 addressing

- Multicast replaces broadcast: FF02::1, FF02::2, and solicited-node groups

🎓 Lab Status: COMPLETED SUCCESSFULLY

---

Cisco CCNA Lab 12.9.2

Configure IPv6 Addresses on Network Devices

---